HTC Vive VR Tracking System Accuracy Measurement

MS Computer Science

Project Report

By:

Jeevitha Pandurangaiah

Pandurangaiahj18@students.ecu.edu

East Carolina University

Spring 2020

Advisor: Dr. Rui Wu

Committee Members:

Dr. Qin Ding

Dr. Zhen Zhu

Dr. Rui Wu

HTC Vive VR Tracking System Accuracy Measurement

By

Jeevitha Pandurangaiah

APPROVED BY:

DIRECTOR OF THESIS:			_______________________

Rui Wu, PhD

COMMITTEE MEMBER:			________________________

Member 1, PhD

COMMITTEE MEMBER:			________________________

Member 2, PhD

CHAIR OF THE DEPARTMENT 		________________________

OF COMPUTER SCIENCE:			Venkat Gudivada, PhD

DEAN OF THE 					________________________

GRADUATE SCHOOL:				Paul J. Gemperline, PhD

TABLE OF CONTENTS

Acknowledgments	4

Abstract	5

1. Introduction	6

2. About VR	8

2.1 Background	8

2.2 Motivation	8

2.3 Purpose of Virtual Reality	8

3. Related Work	9

4. Working of VR device and Vector Nav Sensor	10

5. Experimental Setup	13

6. Detailed Working Flow of the System	15

Step 1: Collecting the data from VR Controller and sensor	15

Step 2: Conversion of Timestamps to the desired units	16

Step 3: Conversion of Quaternions into Euler angles	16

Step 4: Plotting the Graphs of converted values	17

Step 5: Phase unwrapping and Re-sampling	18

Step 6: Error Calculation	20

7. Analyzing the Results	21

8. Future Work	23

9. Conclusion	23

10. References	24

TABLE OF FIGURES

Figure 1: HTC VIVE VR system [6]	11

Figure 2: VECTOR NAV sensor [7]	12

Figure 3: Different views of the IMU sensor mounted on a VR controller	13

Figure 4: Yaw, pitch and roll rotations on HMD [8]	14

Figure 5: Yaw, pitch, roll rotations of VR device	17

Figure 6: Yaw, pitch, roll rotations of IMU sensor	18

Figure 7: Yaw, pitch, roll rotations of VR device after phase unwrapping	18

Figure 8: Yaw, pitch, roll rotations of IMU sensor after phase unwrapping	19

Figure 9: Roll rotations before update	20

Figure 10: Roll Rotations after update	21

Figure 11: Pitch angle from HTC VIVE VR and IMU reference [1]	22

Figure 12: Pitch angle difference from HTC VIVE VR and IMU reference [1]	22

# Acknowledgments

I would like to express my deep gratitude to Professor Dr. Rui Wu, my research supervisor, for his patient guidance, enthusiastic encouragement and useful critiques of this research work. I would also like to thank Dr. Zhen Zhu, the great knowledge resource on sensors, for his constant advice and assistance in keeping my progress on schedule. My heartfelt thanks are also extended to Mr. Grayson Morgan Blankenship and Mr. Christopher Xavier Castro, for their help in conducting the experiments and collecting the data.

I would also like to extend my thanks to the technicians of the laboratory of the CS department for their help in offering me the resources in running the program.

Finally, I wish to thank my family for their support and encouragement throughout my studies.

# Abstract

HTC VIVE Pro is an inexpensive VR system that offers the room-scale tracking up to 11.5 x 11.5 ft area and high accuracy tracking static objects. However, the tracking accuracy can be significantly affected by the number and position of lighthouses and motions of the tracking object. Even though HTC seems to have good accuracy in entertainment, it is not accurate enough in the high precision required scientific research, such as medical surgery simulation. Hence analyzing Virtual reality accuracy is of utter importance and this is a motivational driving force behind our research. The virtual reality system accuracy might be affected by different lighthouse configurations. Since the angles measured are not accurate from the HTC Vive Lighthouses which locates the receiver by triangulation of signals. The VectorNav VN-200 sensor offers a very high level of accuracy by producing a real-time 3D orientation solution over 360 degrees of motion and is used as the benchmark to calculate HTC Vive tracking errors. To analyze the accuracy of the VR tracking system, it’s necessary to align VR’s coordinate system with that of IMU’s and find the errors by shifting data based on the origin system. The root means square error (RMSE) is calculated to show the HTC Vive tracking errors based on Sensor IMU collected dataset. The calculated RMSE value for roll angular rotations was found to be 1.59134. This RMSE value which is found to be smaller proves that VR angular rotations (observed values) are in close agreement with the Sensor IMU data (predicted values).

# 1. Introduction

Though VR is widely used in the gaming industry, product design, and real estate, it has been a challenge in the medical field. For example - It’s been used as a means to train medical students for virtual surgery where the VR controllers must have precision in pointing the virtual patients. It allows them to perform essential procedures in a virtual, controlled environment. Students perform surgeries on virtual patients, which allows them to acquire the skills needed to perform surgeries on real patients. It also allows the students to revisit the surgeries from the perspective of the lead surgeon. On the other hand, if we track the VR controllers used in virtual surgeries, it would be helpful to analyze with what accuracy the surgeries took place. Hence, tracking objects is one of the important applications of Virtual Reality.

The tracking of the VR system appears stable and fast in case of playing games [2]. However, it needs sufficient accuracy and precision to fulfill working of the VR system in the medical field. Here is the experiment with VR tracking systems which has a device that generates a tracking signal, a sensor that detects the signal and a control unit that collects user interaction data. Once the data is collected by the experiment, there are ways to improve the VR tracking system by data massaging. Also, information is represented in the form of a graph to better understand the data by data visualization. This would make easier for the human brain to understand, and visualize the data and detect patterns, trends, and outliers in groups of data.

The lighthouse in the HTC Vive uses rotating mirrors to reflect the IR beams which sweep the room with the beacons of IR light [4]. By determining the difference between the flash and the sweeps and by triangulation of these signals the position of the receiver is identified. The accuracy of the angles that are obtained from the triangulation process depend upon a consistent and high rate of rotation of the mirrors and also precise time measurement. But there is an inherent inconsistency in the rate of rotation of the mirrors due to the type of motors used in the lighthouse. Due to this, the angles measured are not accurate from the HTC Vive Lighthouses. Also, in cases where the view of both lighthouses is lost, the Vive’s screen turns blank and the Vive reports unchanging position and orientation values instead of continuing to update based on the measurements from the inertial measurement units (IMU). During the whole experimental setup, even if one of the lighthouses loses connection, the experiment wouldn’t be successful. Even if the setup is all up to date, the angles might not be accurate from HTC Vive lighthouses.

In this experiment set up, we used a Vector Nav sensor (VN-200) and HTC Vive VR controller tracking system to collect and evaluate the orientational data. We selected VN-200 Rugged sensor due to its best SWAP-C, high-performance Inertial Navigation System (INS) that combines inertial sensors and a high-sensitivity GPS receiver providing optimal estimates of position, velocity, and orientation which is designed for easy testing and integrations [9]. The orientational data collected would be 3 Degree of Freedom where the axis would be yaw, pitch, and roll. The accuracy and precision of the VR controller tracking system deviate from perfection when compared with that of the VN sensor which is known to provide accurate results. Hence, we have made an effort to minimize the error produced by the VR controller mathematically.

# 2. Background

With VR, the user is isolated from the real world while immersed in a world that is completely fabricated. Virtual reality is most commonly used in entertainment applications such as video gaming and 3D cinemas in ancient times. Consumer virtual reality headsets were first released by video game companies in the early-mid 1990s.

The results showed that using a virtual reality application in education increased the learning motivation of students. The attention, satisfaction, and confidence factors of motivation were increased, and these results were found to be significant. Desire to face the challenge in solving the unsolved problems, i.e., concern over practical problems initiates research; Desire to get the intellectual joy of doing some creative work; Desire to be of service to society; Desire to get respectability.

Virtual reality technology is used to create immersive experiences that can help educate and even entertain consumers. Apart from games and entertainment, it's long been used for training airline pilots and surgeons and for helping scientists to figure out complex problems such as the structure of protein molecules. Using VR exposure therapy, a person can face a traumatic event. It has also been used to treat anxiety, phobias, and depression. It is of great help in many medical fields like Dental, Psychiatry if implemented accurately. Virtual reality technology can provide a safe environment for patients to come into contact with things they fear, whilst remaining in a controlled and safe environment.

# 3. Related Work

Niehorster’s research team [2] measured the latency of how the physical movement of the motion sensor reflected in Vive HMD. To prove that their results are accurate, they conducted a test in two identical Vive systems with varying size setups. For comparison, they used a research-grade WorldViz Precision Position Tracking (PPT) system while we used a Vector Nav sensor. They found that the results were consistently observed in all their tests. They used several setups in the tests where tracking quality of all their setups, including the largest where the lighthouse units were placed 7.45 m apart, seemed equally good. They found that VIVE exhibited 2 problems. First, the VIVE uses a reference plane that is tilted away from the ground plane which is causing incorrect roll and pitch measurements. Second, their results suggest that the tilt of this reference plane changed each time the headset lost and regained the tracking of HMD. This can be avoided if the user walks in a small area or not blocking the sensors. They concluded from their experiments that if there is no risk of losing track then a small calibration before running an experiment may enable collecting data of sufficient quality. Also, the error in pitch and yaw measurements may not always play a role because researchers may not use all of the tracker’s measurements.

In Yang’s research team’s study [3], the accuracy and drift measurement in the Vive’s tracking was compared to the Phase Space motion capture system. The tracking accuracy was measured by comparing a set of positions measured with the Vive to positions triangulated using two laser rangefinders and to positions recorded with a research-grade Phase Space motion capture system. The methods are similar to Niehorster et al. with some alterations to allow higher precision and a reduced possibility for human error. They placed four cameras in the corners of the tracking area. Each LED must be seen by at least two cameras for its position to be determined via triangulation. They performed four experiments. Experiment 1 measured the difference between the manually measured position of a Vive tracker using lighthouses and the position reported by the SteamVR plugin in Unity3D in two different sizes of tracking area. The resulting error between the expected position and the position reported by VIVE was less than 1 cm on average. Experiment 2 measured the drift in the position of the Vive HMD and controllers when tracking was lost at irregular intervals which were performed in both the larger and smaller the tracking area. The absolute RMS error was lower in the tracking area for each tracked object. Relative RMS error was much lower than absolute RMS error in the large tracking area, but there was no much information deduced at smaller tracking area. Experiment 3 measured the drift of the HMD, controllers, and a tracker over for 20 minutes by looking after in such a way that there is no loss of tracking. In this setup, the HMD drifted by just 0.44mm, but the right controller drifted by almost 3mm. Experiment 4 compared the position reported by the Vive to the position reported by the Phase Space system. All the data analysis was performed using python libraries and visualizations were created using the Matplotlib Python library. From the analyses of plots, it was clear that the amount of tilt in the ground plane of both tracking systems is similar.

# 4. Working of VR device and Vector Nav Sensor

VR devices are the hardware products used for VR technology to happen. The hardware produces stimuli that override the senses of the user based on the human’s motion. The VR hardware accomplishes this by using sensors for tracking motions of the user such as button presses, controller movements, eye and other movements of body parts. It also considers the physical surrounding world because only engineered hardware and software do not constitute the complete VR system. The users and their interaction with the hardware are equally important. In this experiment, the main focus is on VR controller rotations and the lighthouses.

The VR system consists of a Head-mounted display, two base stations, right controller and left controller as shown in Figure1 [5]. The base stations or lighthouses which are part of the Virtual reality system are small rectangular objects placed in the tracking area. The lighthouse accurately tracks the position and orientation of the user's head-mounted Display and controllers in real-time. It enables the users to move anywhere and re-orient themselves in any position within the range of the SteamVR Base Stations. It is a key technology that enables SteamVR to create a full-room experience in Virtual Reality.

Figure 1: HTC VIVE VR system [6]

Each Base Station contains an IR beacon called Sync Blinker and 2 laser emitters that spin rapidly. 60 times per second, the Sync Blinker would emit a synchronization pulse and 1 of the 2 spinning lasers would sweep an IR beam across the room. The receptors, HMDs, and controllers are covered with photosensors that recognize the synchronization pulse and the laser beams. When it detects a synchronization pulse, the receptor starts to count until one of its photosensors is hit by the laser beam. Lighthouse calculates when the photosensor is hit by the laser and where that photosensor is located to find the exact position of the receptor in relation to the Base Station. When there are 2 Base Stations, the position and the orientation of the receptors in the 3D space of the room is established.

Vector Nav (as shown in Figure2) produces miniature inertial navigation sensors that output attitude (orientation), position, and velocity data. Vector Nav's VN-100 and VN-200 can be used in a wide variety of industrial and military applications. It can also be used in camera/platform stabilization, guided munitions, heavy machinery monitoring, augmented and virtual reality, robotics, primary/secondary flight navigation, and flight simulation.

Figure 2: VECTOR NAV sensor [7]

In our experiment, we used the VN-200 sensor to collect positional and rotational data. The VN-200 is the world's first GPS/INS available in a surface-mount package. The VN-200 incorporates a suite of individually calibrated, MEMS-based 3-axis accelerometers, gyroscopes, and magnetometers, along with a barometric pressure sensor and a high-sensitivity GPS module, providing users with a coupled position, velocity and attitude solution at rates of up to 200 Hz.

# 5. Experimental Setup

The main goal of this project is to collect rotational data from the HTC Vive VR controller tracking system and calculating the errors by knowing how the angles deviate from the Vector Nav Sensor. Firstly, we need to make sure the two base stations are connected, and a room set up is done. Then, both the controller and IMU sensor are mounted /attached tightly as shown in Figure 4 and moved in all the directions in three-dimensional space to capture yaw, pitch, and roll angular data.

Figure 3: Different views of the IMU sensor mounted on a VR controller

The data from the movement of the HTC Vive controller is collected using a custom Visual studio C++ code. Simultaneously the data from the movement of the VectorNav sensor is collected with VectorNav Sensor Explorer software. The motional data is collected by rotating the unit in all the three dimensions in the following way: The unit is rotated around the inertial z-axis to get yaw, rotated about the vehicle-1 y-axis to get a pitch and rotated about vehicle-2 frame x-axis to get a roll. Figure 3 shows how the different rotations can be done on HMD which is applies to Controller as well. The rotational data collected from the VR controller is compared with the data collected from the sensor to identify if the VR controller is rotating accurately w.r.t Sensor. Once the performance is known, we can think about the ways to improve the HTC Vive tracking system to achieve accuracy.

Figure 4: Yaw, pitch and roll rotations on HMD [8]

For this project, we did 3 different experimental setups.

- In the first setup, we rotated the Controller and an IMU to get all the 3 angles which consist of maximum and minimum peaks. We tried to maintain the time gap between each rotation to be 10 secs and repeated it up to 1 second.

- In the second setup, we rotated the Controller and an IMU slowly to get all the 3 angles with a spike which consists of either maximum or minimum peak at a time.

- When we analyzed the data from previous setups, we understood that if we rotate more than 45 degrees, the controller would register a much larger change in degrees than the change that was occurring. Once we confirmed that we are getting stable data, we tried to maintain the rotations within 45 degrees. For a similar reason, we collected pitch data to be < 45 degrees for every rotation. This is our new strategy [1].

The approximate measurements noted in this experiment are as – the height of the two Lighthouses being 66.5 inches, the height of the Controller being 34 inches, the two Lighthouses were 10 feet apart, the Controller position being 4 feet from lighthouse line and 5 feet from each lighthouse. We collected 3 datasets of pitch, roll and yaw changes (9 in total) from the same position. In each dataset, we made sure that we only change one angle at a time but keeping the angle relatively small (30 to 45 degrees). The reference frame being the lighthouse frame, the angular rotations of a controller starts from this reference. The lighthouse line is created by the two connected lighthouses. By observing the datasets collected, we could observe the pattern of 1.0, 0.0, 0.0 and 0.0 for the quaternions when the VR controller (crown facing away from the line) is exactly 90 degrees from the lighthouse line and also when the controller handle is parallel to the ground.

# 6. Detailed Working Flow of the System

## Step 1: Collecting the data from VR Controller and sensor

The initial step is to prepare all the sufficient set up to collect the required data. So, mount the Vector Nav sensor onto the VR Controller with the lighthouse stations at a maximum distance of 5feet from one another. Whenever the setup is ready, start the recording in the sensor explorer software. Simultaneously start the Visual Studio C++ script to get the orientational data from a VR controller. After collecting information about all the required dimensions, stop the Visual Studio as well as sensor explorer. The data from both the software were collected in a CSV format since processing the data in MATLAB would be much easier. The CSV files having a sensor-related data consisting of a timestamp in DateTime format and Euler angles related to rotations while controller related data consisting of a timestamp in POSIX format and Quaternions.

## Step 2: Conversion of Timestamps to the desired units

To compare both the timestamps from a sensor and a controller, both the formats should be the same. Also, it is best to compare two timestamps when it is having a single numeric value such as in seconds rather than DateTime format. The timestamp received from Sensor would be a DateTime and hence has to be converted to a single numeric value which is a POSIX time. The POSIX time or UNIX epoch is the number of seconds that have elapsed since Jan 1, 1970. For example: Consider a DateNum “2019-06-25T14:59:14.124” from a sensor which is converted to POSIX number of 10 digits and 3 decimal digits “1561474754.124” to compare with the controller which also generates 13-digit POSIX number. For this conversion, there is a method in MATLAB called posixtime() using which all the DateTime values in array format can be converted.

## Step 3: Conversion of Quaternions into Euler angles

When the experiment is performed the data captured from the sensor would be in degrees of Euler angles which is best to compare with any angular measure. But the problem which arises here is with the VR controller since it gives out Quaternions. Therefore, to compare the Sensor and controller, one of the measures has to be converted. In this project, converted Quaternions to Degrees. A Quaternion is a four-part hyper-complex number used in three-dimensional rotations and orientations. A Quaternion number is represented in the form a + bi + cj + dk, where a, b, c are real numbers, and i, j, k are basic elements. The Euler angles are three angles to describe the orientation of a rigid body w.r.t to a fixed coordinate system. The first step in the conversion is to extract parts of the Quaternion array one by one from a table. Passing the extracted Quaternion, rotation sequence and rotation type as a parameter to the eulerd() MATLAB function which converts the Quaternion into Euler Angles.

## Step 4: Plotting the Graphs of converted values

After the conversion of values to the desired units, it is ready for us to compare the values using graphs. The timestamp collected from both the devices can never be the same since the computer system has a different time clock. Hence, we can only try to compare how the degrees vary w.r.t timestamp. The graphs would look in this way:

Figure 5: Yaw, pitch, roll rotations of VR device

Figure 6: Yaw, pitch, roll rotations of IMU sensor

## Step 5: Phase unwrapping and Re-sampling

When you carefully observe the above two graphs, you can see a large amount of phase change between the two graphs. When working with a Phase delay, it is often necessary to “unwrap” the phase response Θ. Phase unwrapping ensures that all appropriate multiples of 2pi have been included in Θ. MATLAB has a function called unwrap() which implements a numerical algorithm for phase unwrapping.

Figure 7: Yaw, pitch, roll rotations of VR device after phase unwrapping

Figure 8: Yaw, pitch, roll rotations of IMU sensor after phase unwrapping

The MATLAB function Q = unwrap(P) unwraps the radian phase angles in a vector P. Whenever the jump between consecutive angles is greater than or equal to pi radians, unwrap shifts the angles by adding multiples of +/- 2pi until the jump is less than pi. If P is a matrix, unwrap operates column-wise. If P is a multi-dimensional array, unwrap operates on the first dimension whose size is larger than 1.

When we collected the data from VR Controller and IMU sensor, we found that the frequency of the sensor is different from the frequency of the VR controller. So, we cannot compare both the timestamp data. The solution is to Resample the data. But not at the same rate. Resampling is nothing but a methodology of economically using a data sample to improve the accuracy and quantify the uncertainty of a population parameter. We can resample uniform or non-uniform data to get a new fixed rate. y = resample (x, p, q) resamples the input sequence x, at p/q times the original sample rate.

## Step 6: Error Calculation

This error calculation is based on the new strategy where the data is collected with angular rotations being within 45 degrees.

Below are the steps to calculate errors using excel spreadsheets:

- Collect data from both VectorNav and HTC Vive system.

- Align two coordinate systems. Find out both system (0,0,0) origin points and shift data based on this origin system.

- Calculate yaw, pitch, and roll errors (RMSE).

To simplify the error calculation, we usually select one rotation dataset each time, such as a roll.

Figure 9: Roll rotations before update

This graph implies that the VR and sensor roll rotational data looks very similar but has opposite peaks. So, we multiplied VR data with -1 to rotate or reverse the graph so that both the line graphs align.

Figure 10: Roll Rotations after update

The RMSE values can be calculated with the below formulae in excel:

RMSE=SQRT(SUMSQ(Cell Range of angular diff between VR and Sensor)/COUNTA(Cell Range of angular diff between VR and Sensor))

The calculated RMSE was 1.59134 which shows that the VR device has a minute error w.r.t the roll rotations of the collected data. Thus, some of the results are very good. The issue over here is we considered only roll rotations. When we do rotations with Controller mounted on the IMU sensor intended to get roll rotations, the IMU sensor collects only roll movements while the controller will be mixed with pitch & yaw rotational data as well. This is because of how they have defined coordinate system. We don't want to mix up the data hence we limit to one angular rotation i.e roll.

# 7. Analyzing the Results

These figures 9 and 10 are from the paper [1] and the mathematical model to estimate errors was from Dr. Zhu.

Figure 11: Pitch angle from HTC VIVE VR and IMU reference [1]

As per the experiment conducted, the two lighthouses were set up with sufficient distances apart with an HTC tracker facing perpendicularly to the middle point of both lighthouses. The tracker was manually rotated within a 3 min window with a periodically changing pitch angle. From Figure 9, we can show that the HTC-measured pitch angle (marked as VR) is in close agreement with that from the reference IMU sensor.

Figure 12: Pitch angle difference from HTC VIVE VR and IMU reference [1]

Figure 10 shows the difference between both the measurements and how the VR data deviate from the IMU data. The calculated standard deviation was 0.47 deg from this dataset which shows that the values in a statistical dataset are close to the mean of the dataset, on average. The tracker and the IMU values were manually aligned to reduce the limitations of finding accuracy.

# 8. Conclusion and Future Work

HTC Vive tracking system is not only used by the gaming industry but also by researchers due to its low price, high accuracy and continued adoption of beacon technology. In this project, the data collected initially were found to be unstable. The rotational error from a VR controller was found by data comparison with Sensor IMU mounted on it. On careful observations, we realized that there are large changes in VR controller rotations which made the data analysis difficult. Once we found the latest strategy to get stable data by rotating within 45 degrees, we could calculate the rotational error of a VR device. The sensor IMU was used as a benchmark to find the performance of a VR device. When we calculated the RMSE value for roll rotations, which is being a smaller value i.e 1.59134 implied the accuracy of an HTC VIVE VR system. The drawback is that when roll movements are performed, the VR controller mixes it with pitch and yaw rotational data. We are ignoring them and considering only roll angular rotations.

In our experiment, we use only two lighthouses in the setup. There might be a problem with blindspot when we use just two lighthouses. During the experiment, the human may even block the device’s photosensors which would affect VR accuracy. In the future, the same experiment can be repeated by employing more than two lighthouses assuming it might improve the accuracy. Also, we would plan to build an error prediction model where the preliminary work has already been introduced in our paper [1].

# 10. References

- Rui, W., Jeevitha, P., Grayson, M. B., Christopher, X. C., Shanyue, G., Andrew, J., & Zhen, Z. (2020). Evaluation of Virtual Reality Tracking Performance for IndoorNavigation. (Accepted by IEEE/PLANS 2020)

- Niehorster, D. C., Li, L., & Lappe, M. (2017). The accuracy and precision of position and orientation tracking in the HTC vive virtual reality system for scientific research. IPerception, 8(3), 1–23. https://doi.org/10.1177/2041669517708205

- Yang, Y., Weng, D., Li, D., & Xun, H. (2017). An improved method of pose estimation for lighthouse base station extension. Sensors (Switzerland), 17(10), 1–14. https://doi.org/10.3390/s17102411

- Williams, E. (2016). Alan yates: why valve’s lighthouse can’t work | Hackaday. In Alan yates: why valve’s lighthouse can’t work | Hackaday. Hackaday.

- MediaWiki. (2015). Lighthouse | xinreality. Lighthouse | xinreality. Xinreality.

- Figure 1. IANS. 2018. HTC Vive Business Edition VR headset launched for Rs 1,26,990. (2018). Retrieved 23 February 2018. Available at https://www.91mobiles.com/hub/htc-vive-business-edition-vr-headset-india-launch-price-details-features-specifications/.

- Figure 2. Next-generation MEMS inertial navigation. vectornav. Retrieved from https://www.vectornav.com/.

- Figure 4. Dupin, L. VR is Ready for Consumers, are You Ready for It? Awwwards.com. Retrieved 17 March 2019, from https://www.awwwards.com/vr-is-ready-for-consumers-are-you-ready-for-it.html.

- Kok, M., Hol, J. D., & Schön, T. B. (2017). Using inertial sensors for position and orientation estimation. Foundations and Trends in Signal Processing, 11(1–2), 1–153. https://doi.org/10.1561/2000000094.